mapleMaker Mini V2 Getting Started Guide

Table of Contents:

1. Firmware Configuration
a. Installing & configuring the  Arduino IDE

b. Configuring and uploading the Marlin Firmware

2. Printer Control using Cura
a. Installing and configuring Cura
b. Connecting your printer to Cura

c. Understanding the print controller

d. Validating your printer capabilities

i. Axis movements

ii. End stop movement

3. Final printer configuration

a. Setting your end stops 

b. Leveling your print bed

c. Loading Filament

d. Extruding Filament

e. Understanding the Printer LCD

4. Your first Print

a. Understanding the Cura Interface

b. Loading and slicing your first model

c. Starting & completing a print

5. Troubleshooting 

a. Hardware

i. Axis/Extruder moves in wrong direction

ii. Endstops do not respond

iii. Extruder/Bed doesn’t move or doesn’t move precisely 

iv. Extruder does not extrude filament

v. First layer is too high

b. Software

i. Printer doesn’t connect

ii. Driver details for OSX & Windows 8.1
Note: While every attempt has been made to verify the accuracy of this document, there may be differences in this document that does not reflect the current state of software or applications. Please always consult with the original application developer to obtain the latest release of software. While this document contains images taken from a Windows PC, the process is primarily the same for OSX users unless otherwise noted. Linux users may need to consult each application’s documentation for further guidance. 

First Power up & Verification

Now that we’ve assembled our printer, our first goal before proceed further is to validate that the printer functions. To do so, connect the power supply to the printer. You should see a row of blocks on the LCD display of the printer, as well as see the Hot-End cooling fan, and electronics cooling fans working. There should be no smoke, flames, or other effects at this time. Congratulations!


Firmware Configuration

a. Installing & configuring the  Arduino IDE

Before we begin configuring the firmware, we must first install the appropriate software and drivers to interact with our printer. To begin, we must first install the Arduino IDE. Please visit: http://www.arduino.cc/en/Main/Software and select the latest IDE for your platform. [image: image1.png]Buy Download Products ~ ng ~ Forum  Support v Blog IN  SIGN UP

Windows installer
ARDUINO 1.6.4 Windows zIP file for non admin install
il Arduino
de and upload it to

s, Mac 05 X

Mac OS X107 Lion or newer

] uX 32 bits

Linux 64 bits

Release Notes
Source Code
Checksums.





Once you have downloaded the appropriate IDE, run the installer and follow the commands on screen. This installation will also include the driver needed to connect to your printer in later steps. 

Once you have installed the Arduino software, you may now connect your printer to the power supply, and connect the USB Type B cable from your computer to the printer. 

We must now configure the Arduino software to communicate with our printer, to do so, launch the Arduino IDE. From the Tools Menu, select the Board option, and then the ‘Arduino Mega 2560 or Mega ADK’. The Mega 2560 is the core of our printers electronics, and is commonly used in the RAMPS 1.4 configuration. 

[image: image2.png]sketch_may20)

| Teols| Help.
Auto Format
Archive Sketch
Fix Encoding & Reload
Serial Monitor

Board
Serial Port

Programmer

Burn Bootloader

CtrlsT

Ctrl+Shift-M

Arduino Uno
Arduino Duemilanove w/ ATmega328

Arduino Diecimila or Duemilanove w/ ATmegal68
Arduino Nano w/ ATmega32s

Arduino Nano w/ ATmegal6B

Arduino Mega 2560 or Mega ADK




From the Tools menu again, select the Serial Port. In this drop down you will see several options. The option that we wish to use should begin with ‘/dev/tty.usbmodem…’. This should be the location of our printer. It is important that you do not have any other Arduino powered devices connected to your PC at this time. 
[image: image3.png]>
[l /dev/tty.Bluetooth-PDA-Sync
/dev/cu.Bluetooth-PDA-Sync

Programmer > /dev/tty.Bluetooth-Modem
Burn Bootloader

/dev/cu.Bluetooth-Modem

/dev/tty.usbmodemfd131

/dev/cu.usbmodemfd131





Congratulations, you’ve installed the Arduino IDE and configured it to talk to your printer. In the next step we will begin modifying the firmware and writing it to our printer. 

b. Downloading & configuring the Marlin Firmware

Now that we have the Arduino software configured and connected to our printer, we must obtain the Firmware and make the necessary changes to it in order to suite our printer. The firmware is the ‘brains’ of the printer, which allows it to interpret gcode, commands, and control motors, extruders and all kinds of accessories. For our purposes we will be using one of the most popular firmwares, MarlinFirmware.

Visit https://github.com/MarlinFirmware/Marlin. On the right hand side of the window, choose the ‘Download ZIP’ button to download the latest release of the Marlin Firmware.  
[image: image4.png]B Marlin

B giignore

B wravisyml

B COPYING.md

B README md

=2 README.md

Update language_pth

Update gtignore

Commented out configs that are failing with no easy fixes for the moment
replaced ASCIl GPL v3 license by markdown version

More authors.

2 days ago
5 months ago
5 months ago
4months ago

6 days ago

HTTPS cone URL
netps://github.contna I

‘You can clone with HTTPS or
Subversion. @

Clone in Desktop

% Download ZIP




Once the download has completed, extract the Zip folder to a safe location on your computer. 

There will be several sub directories and documents contained within the download. For our purposes, we are concerned with the files and configurations located at:  Marlin-Release\Marlin-Release\Marlin. 
With the Arduino IDE open, select the File > Open option, and then navigate to the directory listed above. In this directory, locate the ‘Marlin.ino’ file, and click ‘Open’.

[image: image5.png]File Edit Sketch Tools Help

sketch_may20a

£ Open an Arduino sketch..

i Marin - 0frE

Name Date modified
* LiquidCrystlRus.cop 5/18/2015 527 AM
[ UquidCrystalRush 5/18/2015'527 AM
[ Mokefile 5/18/2015'527 AM
[ Merlinh 5/18/2015 527 AM

[[E]Mariin.no 5/18/2015 527 AM
[ Merlin pae 5/18/2015 527 AM
++ Marlin_main.cpp 5/18/2015'527 AM

4 MarinSeralcpp. 5/18/2015 527 AM
[ MarinSeralh 5/18/2015 527 AM
4 motion_control.cpp 5/18/2015 527 AM
- motion_controlh 5/18/2015 527 AM
ipinsh 5/18/2015 527 AM

olam S/18/2015 527 AM





With the Firmware project files loaded, your IDE should look like this: 

[image: image6.png]warin | BlinkM.opp | BlinkMh | Configurationn | ConfigurationStore.cpp | ConfigurationStoreh | Configuration_acih | D il
T Y] -
e

Reprap firuware based on fprinter and grbl.
Copyright (€] 2011 Camiel Gubbels / Erik van der Zaln

This progran is fres software: you can redistribute it and/or modify
it under the terms of the CNU General Public License as published by
the Free Software Foundation, either version 3 of the License, or
(at your option) any later version.

This progran is distributed in the hope that it will be useful,
bt UITHOUT ANY WARRANTY without even the implied warrancy of
MERCHANTABILITY or FITHESS FOR A PARTICULAR PURPOSE. e the
CNU General Public License for more details.

You should have received a copy of the GNU General Public License
along vith this program. If not, see <http://wuw.gmu.org/licenses/>.
s

e
This firmvare is a mashup betveen Sprinter and grbl.
(hteps: //gi thub . con/kLinent/Sprinter)
(hteps: //github. con/sinen/grbl /tres)





In the tabs at the top of the window, locate the tab ‘Configuration.h’. This is where we will make all the necessary changes required for our firmware. 

The first configuration we must modify is the defined motherboard. In our case, this is located on line 47 of the configuration file. 

Note: The line number is located in the lower left hand corner of the Arduino IDE. 

Change the defined motherboard from #define MOTHERBOARD BOARD_ULTIMAKER to #defineMOTHERBOARD 33. 33 denotes the stock RAMPS 1.4 setup used in our printer. 

The second option we must modify is located on line 107. This is the thermistor used in the hot-end. For our purposes, if using an e3D Lite6, this value should be changed from -1 to 5. Set the values of line 108-110 to 0. Your Thermistor table should look like so: 

[image: image7.png]"
7
7
7
7
7
7
7
7
7
7
7
7
7
7
7
7

10
1
12
13
20
60

51
52
55

1x

100k RS thermistor 198-961 (4.7k pullup)

100k beta 3950 1% thermistor (4.7k pullup)

100k 0603 SHD Vishey NTCS060SESLO4FXT (4.7k pullup) (calibrated for Nakibox hot bed]
100k Hisens 3950 1% up to 300°C for hotend "Simple ONE " & "Hotend "ALL In ONE"
the PTLO0 circuit found in the Ultinsinboard V2.x

100k Haker's Tool Works Kapton Bed Thermistor beta=3350

ohm pullup tebles - This is not nornal, you would have to have changed out your 4.7k for lk
(but gives greater accuracy and more stable PID)

100k thermistor - EPCOS (Lk pullup)

200k thermistor - ATC Semitec 2045T-2 (Lk pullup]

100k thernistor - ATC Semitec 1045T-2 (Used in ParCan a J-Head] (Lk pullup]

1047 13 PELO00 with 47 pullup
1010 15 L1000 with Lk pullup (non standard]
147 13 PLl00 with 47 pullup

110 15 P00 with Lk pullup (non standard)

/7 This mekes temp sensor 1 a redundant sensor for sensor 0. If the temperatures difference betveen these sensor
J/#define TENP_SENSOR_1_AS_REDUNDANT




We must now configure our maximum extruder temperatures. Because we are using an e3D Lite6, our maximum temperatures cannot exceed 250. Change this value on line 132 like so:

[image: image8.png]// Actual temperature must be close to target for this long before M109 returns success
#define TENP RESIDENCY_TINE 10 // (seconds)

#define TEMP_HYSTERESIS 3 77 (deqC) renge of +/- temperatures considered "close” to the target one
#define TEWP VDNDOW 1 77 (4eqC] Vindow around target to start the residency timer x degC early.

/7 The mininal temperature defines the temperature below which the heater will not be ensbled It is used
77 to check that the viring to the thermistor is not broken.

/7 Otherwise this would lead to the heater being povered on all the time.

#define HEATER_O_NINTEHP 5

#define HEATER_L NINTEHP 5

#define HEATER 2 NINTEP 5

#define BED_MINTEP 5

/7 When temperature exceeds max temp, your heater will be switched off.
/7 This feature exists to protect your hotend from overheating accidentally, but *HOT* from thermistor short/fa:
/7 You should use NINTEHP for thermistor short/failure protection.

77 £ your bed has low resistance e.g. .6 ohm and throvs the fuse you can duty cycle it to reduce the
77 average current. The value should be an integer and the heat bed will be turned on for 1 interval of
77 HEATER_BED_DUTY_CYCLE_DIVIDER intervals.

J/#define HEATER_BED_DUTY CYCLE_DIVIDER 4




Our next configuration is defining the end-stops. Our printer uses end-stops at the minimum positions, and none at the maximum, so we must remove the maximum end-stop configuration from the firmware. In these cases 2 forward slashes like so // will comment out any line of code and render it un-usable to the Arduino. You will need to comment out lines 284, 285, and 286 like so: 
[image: image9.png]/7 coarse Endstop Settings
#define ENDSTOPPULLUPS // Comnent this out (using // at the start of the line] to dissble the endstop pullup res

#ifndet ENDSTOPPULLUPS
77 fine endstop settings: Individusl pullups. will be ignored if ENDSTOPPULLUPS is defined
77 #aefine ENDSTOPPULLUP_XIAX
77 #aefine ENDSTOPPULLUP_YIAX
77 #aefine ENDSTOPPULLUP_ZMAX
77 #define ENDSTOPPULLUP_XNIN
77 #define ENDSTOPPULLUP_YNIN
77 #aefine ENDSTOPPULLUP_ZMIN

#enait

#ifdet ENDSTOPPULLUPS

#define ENDSTOPPULLUP_XNIN
#define ENDSTOPPULLUP_YNIN
#define ENDSTOPPULLUP_ZMIN

#enait

77 The pullups are needed if you directly comnect a mechanical endsvitch betveen the signal and ground pins.
const bool X_MIN_ENDSTOP_INVERTING = trus; // set to true to invert the logic of the endstop.




We must also set the end-stop inverting to false. Change the true value to false for lines 293-298 like so: 

[image: image10.png]#endif

#ifdef ENDSTOPPULLUPS
//#define ENDSTOPPULLUP_XIAX
//#define ENDSTOPPULLUP_YIAX
//#define ENDSTOPPULLUP_ZMAX
#define ENDSTOPPULLUP_XNIN
#define ENDSTOPPULLUP_YNIN
#define ENDSTOPPULLUP_ZMIN

#enait

/7 The pullups are needed if you directly comect a mechanical endsvitch betveen the signal and ground pins.

//§define DISKBLE_NAX ENDSTOPS
//#define DISKBLE_NIN_ENDSTOPS

/7 Disable max endstops for compatibility with endstop checking routine
#1F defined(COREXY) ce !defined (DISABLE_MAX ENDSTOPS)

#define DISABLE_MAX ENDSTOPS
fendit




Our most important configuration of the firmware is defining the size of the printer’s volume. Defining an area larger than what the printer can physically print is a bad idea, and can cause serious issues, so we must set these values to be equal to or slightly less than the actual physical volume of the printer. The fields that need to be changed are located on lines 337, 339 and 341. In our case, these values should be changed to 140 like so:
[image: image11.png]#define INVERT EO DIR false // for direct drive extruder v8 set to true, for geared extruder set to false
#define TNVERT ELDIR false  // for direct drive extruder v9 set to true, for geared extruder set to false
#define TNVERT E2 DIR false // for direct drive extruder v set to true, for geared extruder set to false

77 ENDSTOP SETTINGS:
77 Sets direction of endstops vhen homing:
#define X _HOME DIR -1
#define Y_HOME DIR -1
#define Z_HOME DIR -1

#define nin_software_endstops true // If ©true, wxis uon't move to coordinates less than HOME_POS.
#define max_software_endstops true // If true, @xis von't move to coordinates greater than the defined lengths belou.

/7 Travel limits after homing

#define X MAX_LENGTH (X _MAX_POS - X_MIN_POS]
#define Y MAX_LENGTH (Y_MAX_POS - Y_MIN_POS]
#define z MAX_LENGTH (2 MAX_POS - Z_MIN_POS]
7 Bed huto Leveling





We must now configure our Homing Feedrate. For the purposes of our printer, these should be set to the following values on line 478: 

[image: image12.png]//Manual howing switch locations:
// For deltabots this means top and center of the Cartesian print volume.

fdefine MANUAL X HOME_POS 0

fefine MANUAL Y _HOME_POS 0

#define NANUAL Z_HOME_POS 0

//#define NANUAL Z HONE_POS 402 // For delta: Distance between nozzle and print surface after homing.

//// MOVENENT SETTINGS
#define NUM_AXIS 4 // The axis order in all axis related arrays is X, ¥, Z, E





After we have defined our feed rate, we must set our default steps per millimeter. These values indicate how many motor steps are required to travel 1mm. A good baseline for our printers is provided below. Modify these values in your configuration on line 482: 

[image: image13.png]// default settings

#define DEFAULT_MAX_FEEDRATE (500, 500, 5, 25)  // {mm/sec)
#define DEFAULT_MAX_ACCELERATION {9000,5000,100,10000}  // X, ¥, 7, E maximm start spesd for a



 

It’s also good to modify our max feed rates to the following on line 483:

[image: image14.png]#define DEFAULT_AXIS_STEPS PER_UNIT  (80.2, 80.2, 4417, 147) // default steps per unit for Ultiudker

#define DEFAULT_MAX_ACCELERATION {9000,5000,100,10000}  // X, ¥, 7, E maximum start spesd for ace




And lastly, set our default max acceleration to the following values on lines 484, 486 and 487

[image: image15.png]$define DEFAULT_AXTS_STEPS_PER_UNIT  (80.2, 80.2, 4417, 147} // default steps per unit for Ultinsker

faefine DEFAULT_MAX_FEEDRATE (250, 250, 2, 22} // {mm/3ec)
fdefine DEFAULT_MAX_ACCELERATION {1000,1000,2,10000)  // X, ¥, Z, E maximm start speed for acceler
faefine DEFAULT_ACCELERATION 500 /X, ¥, Z and E max acceleration in mn/s*2 for printing move

faefine DEFAULT_RETRACT_ACCELERATION SO0 // %, ¥, 7 and © e acceleration in wa/s'2 for retracts




We’ve now completed the most crucial parts of the configuration, but we still have a few options remaining. 
You will also want to define the EEPROM settings for the configuration, namely enabling them. To do so, uncomment (remove the 2 ‘//’) from lines 519 and 522 like so:

[image: image16.png]/4 EEPROM
// The microcontroller can store settings in the EEPRON, e.y. max velocity...

// MS00 - stores parameters in EEPRON

// MSOL - reads parameters from EEPROM [if you need reset them after you changed them temporarily).

/7 MS02 - reverts to the default "factory settings”. TYou still need to store them in EEPRON aftervards if you vant to.
//define this to ensble EEPRON support

#define EEPROM_SETTINGS

//to disable EEPROM Serial responses and decrease program space by ~1700 byte: comment this out:

// please keep turned on if you can.

§define EEPROM_CHITCHAT




Lastly, we must enable support for the LCD panel on our printer. To do so, uncomment the line 552. This will enable support for the LCD panel included with your printer. 

[image: image17.png]// The RepRapDiscount Smart Controller (white PCE)
// http://reprap. orq/viki/RepRapbiscount Smart Controller

// The GADGETSSD G3D LCD/SD Controller [blue BCEH)
// Bttp://reprap. orq/uiki/RAMPS 1.3/1.4 GADGETSSD Shield with Panel
//#define G3D_PANEL





Click File > Save when you are finished. 

Congratulations! Your firmware is now complete and ready to be verified. To verify and compile your firmware, click the check icon in the upper left corner of the window. If all goes correctly, in a few moments you should receive a message at the bottom of the screen indicating that the firmware has been compiled like so:

[image: image18.png]Marlin | Blinki.cpp | Blinkih | Confgurationh | ConfigurationStore.cop | ConfigurationStoreh | Configuration_advh | DOGMbitmaps.h | Lia ¥ Cn

//#define LCD_FEEDBACK_FREQUENCY_HZ 1000 J/ this is the tone frequency the buzzer plays vhen on UI feedback. ie Screen Click +
//#define LCD_FEEDBACK_FREQUENCY_DURKTION_MS 100 // the duration the buzzer plays the UI feedback sownd. ie Screen Click

// The MaKr3d Makr-Panel with graphic controller and SD support
// http://reprap. orq/viki/MaKe3d MakrPanel
//#deTine NAKRPANEL

// The RepRapDiscount Smart Controller [vhite PCH)
// Bttp://reprap. orq/viki/RepRapbiscount Smart Controller
#define REPRAP_DISCOUNT_SHART_CONTROLLER

// The GADGETSSD G3D LCD/SD Controller [blue BCEH)
// Bttp://reprap. orq/uiki/RAMPS 1.3/1.4 GADGETSSD Shield with Panel
//#define G3D_PANEL

// The RepRapDiscount FULL GRAPHIC Smart Controller [quadratic white PCB)
// http://reprap. orq/viki/RepRapbiscount Full Graphic Smart Controller
'

// ==> REMEMBER TO THSTALL USglib to your ARDUTHO library folde
//#define REPRAP_DISCOUNT_FULL_GRAPHIC_SHART CONTROLLER

Bttp: //code. google. con/p/uBylib /uiki /usylib

// The RepRapiorld REPRAPVORLD_KEYPAD vl.1
// http://reprapvorld. con/zproducts detailssproducts id=202ccPath=159] 1626

//#define REPRAPUORLD_KEYPAD

//#define REPRAPVORLD_KEYPAD_MOVE_STEP 10.0 // how much should be moved vhen a key is pressed, eg 10.0 means 10um per click 2
<[ I ] ’

Done compiling,

Arduin




With the firmware compiled, and your printer connected to your PC, you may begin the process of uploading the firmware. To do so, after compiling the firmware, click the arrow button next to the Compile button. The message in the bottom of the window will change to ‘Uploading Firmware’ and you will see a progress bar appear. When the firmware is flashed correctly, you will see ‘Upload finished’ appear in the bottom of the window, and you should see your printer information appear on the LCD panel of your printer. Congratulations!



Printer Control Using Cura


a. Installing and configuring Cura

Now that we have compiled and uploaded the firmware for our printer, we must turn our attention to the software we will use for manipulating the models and creating profiles that allow us to print objects. These is commonly known as a slicer, and there are many options available on the market today. In keeping with our open-source nature, we are using Cura, developed by the amazing team at Ultimaker. 
In order to begin, please visit: https://software.ultimaker.com/ and download the latest stable release of Cura for your platform.

[image: image19.png]Software downloads

These are official releases of the Cura software for use with your Ultimaker.
Cura is Open Sousce Software and sources can be found at github.

Below you can download the latest release for your operating system (Windows. MacOS X or Linux)

2C

Download Cura_15.02.1 for WinNT

See below for other versions.





Once downloaded, launch the installer and follow the prompts on screen to install Cura. At the end of the installation Cura will automatically launch, and the first run wizard will appear. Click Next to continue. 
[image: image20.png]Add new machine wizard

This wizard wil help you n setting up Cura for your machine.





On the next screen you will be asked to select the type of printer that you intend to use. For our purposes, choose the last option, Other, and click Next. 

[image: image21.png]Select your machine

Wihat kind of machine do you have:

e Original
Jtimaker Original+

ther (Ex: RepRap, MakerBot, Witbox)

The collcton of anonymous usage information helps with the continued improvement of Cura.
This does NOT submit your models onlne nor gathers any privacy related information.
Submit anonymous usage information

For ful details see: https ki utmaker.com/Curaistats





The following screen presents a number of different printer configurations for us to choose from. We will choose the final option, Custom. Click Next to Continue. 

[image: image22.png]Other machine information

The folowing pre-defined machine profies are avalable

Note that these profies are not guaranteed to give good resiits,
or work at al. Extra tweaks might be requred.

Ifyou find issues with the predefined profies,

or want an extra profie,

Please report it at the github fssue tracker.

ers
DeltaBot
MakerBotRepicator
Mendel

ord

Prusa Mendel i3
Rigd:D

RigidBot

RigidBottig

Witbox

Zone3d Printer

juia

pundhtec Connect XL

Custom.





We will now need to enter some basic information about our printer. For our printer, these settings are as follows. Click Finish when you ready. 
[image: image23.png]Custom RepRap information

RepRap machines can be vastly different, so here you can set your own settings.
Be sure toreview the defauit profile before running it on your machine.

If you like 2 default profile for your machine added,

then make an issue on github,

You wil have to manually nstal rin or Sprinter frmware.

Machine name. mapleMaker Mini

Machine width X (m) 140
Machine depth Y (mm)

Machine height 2 (mm) 140
Noze size () 04

Heated bed [m]
Bed centeris 0,0,0 Rostock) []





With the basic configuration complete, we will spend a few minutes tweaking our slicing profile to suite our first print. 

The primary interface of Cura is broken into 2 components. To the right we have the virtual build volume. This where your models are placed. The box on screen represents the build volume of your printer. The second component is the slicing parameters. These are the user controllable options that adjust variables such as the layer height, print speed and support material. 

[image: image24.png]e L R—
P T—

Speed and Temperature

T S—

Printng temperatire () 210
Support

swee e
B e
Filament

P

ot





For the purpose of our first prints, lets adjust some of these settings. 

First and foremost, we will adjust the layer height from the stock 0.1 to 0.3. The lower this number, the thinner the layers will be. This will produce a better print, but can also be more challenging and time consuming. 0.3 layer heights are a good compromise between ease of use and quality. 

[image: image25.png]Layer height (mm)

Shel thickness (vm)

Enable retraction





The second option that we must modify is the Filament diameter. This value should be changed to 1.75mm, the diameter of the filament that our printer uses. 
[image: image26.png]Diameter (nm)
Flow (%)





To initially test our printer, we will need to load the advanced UI and Pronterface UI for Cura. To do so, from the File Menu in Cura, choose Preferences. In the Preferences window, change the Printing Window Type to Pronterface UI like below:

[image: image27.png]Print window
Printng window type
Colours

Model colour

Language
Language

Pronterface Ul _v.

Basic
[]

Flament settings
Density (kg/m3)

Cost (rice/kg)

Cost (ricefm)

SD Card settings

Auto detect SD card drive
Base folder to replicate on SD card |C:\Users|Ryan/Doc]

Cura settings

Check for updates
Send usage statistics

ok




For the time being, the rest of the default values will work for our first prints. We will also cover these options in depth later in this guide. 

You can also learn more about Cura through Ultimaker’s official documentation at: https://ultimaker.com/en/support/view/248-cura-manual
b. Connecting your printer to Cura

Connecting your printer to Cura couldn’t be easier. Simply connect the USB cable from your PC to the printer, ensure that your printer is powered up, and launch Cura. You should see the middle icon in Cura’s 3d stage labeled ‘Print with USB’. 

c. Understanding the print controller

For the following guide, we will first need to load a temporary model into the stage of Cura in order to display the printer controller. We will however not be printing at this moment. 
To load a model, click the folder icon on the stage, and select your model. For illustration purposes, we are using a extruder base from the mapleMaker Mini printer. 

[image: image28.png]le Tools Machine Expert Help

Basic [ advanced | Plugin | Start/End-GCode

Quality

Layer height (mm)

Shel thickness (vm)

Enable retraction

Fll
Bottom/Top thickness (m)

Fil Density (%)

Speed and Temperature

Printspeed (nm)s) )

Printng temperature (€) 210

Support

Support type

Platform adhesion type.

Flament

Diameter (nm)

Flow (%)

@ v 1« Maplebrint Mini V2 » STL Vo] | serchst

Organize = Newfolder
Ui MsplePrint Mini 2
Ui Printer_Build_Docs
Ui Keyboard
Ui ps0
5 nventor
Ui Visusl Stucio 2013
b cao

@ OneDrive
& Homegroup

M This PC

File name:

Neme
(<] MPMV2_Crossmember 3.ti

2] MPMV2_Crossmember 4.ti

<] MPMV2_Crossmember 5.ti

2] MPMV2_Crossmember 6.ti

(] MPMV2_Crossmember 7.sti

2] MPMV2_Crossmember 8.ti

(] MPMV2_Extruder Base.stl

(] MPMV2_Extruder_Fan_Mount.st

(] MPMV2_Extruder Mount Platest

(] MPMV2_Extruder Mount Plate E30.st1

] MPMV2_Extruder V2R Base.stl
v«

/13/20158: ¥
>

MPMV2_Extruder Base.stl Al €5t 0" da” amf;" by v

Open

Cancel





You should now see the object placed in the centre of the stage. When ready, click the Print with USB button. You should now see the Pronterface Interface:

[image: image29.png]i

250 Temperature

b:: o ]

ﬁd“ Bed temp

0 I
=

B





The Pronterface interface is broken into 4 main areas. The first area is the motion controllers for the printer itself. The large cirular dial allows you to control individual axis’ of the printer in 0.1, 1, 10 and 100mm increments. 

[image: image30.png]



The vertical sliders to the right of the axis controller allows the user to raise or lower the Z axis, as well as extrude filament through the extruder. Note: Do not extrude or retract filament unless the hot end is up to temperature. Doing so will damage your printer. 

[image: image31.png]L =LDN




The third area is the temperature control and information. The graph will display the active temperature of the hot end, while the adjoining fields will allow the user to modify the temperature of the extruder. 

[image: image32.png]Temperature

b:: . &
5““ Bed temp

o o =

5]





The last area is the gcode window. This window will display various gcode information such as gcode for your sliced models, as well as error messages and other information. At the bottom of the window is a single line field which will allow the operator to send commands to the printer. We will be using this for our first test runs. 

[image: image33.png]



d. Validating your printer capabilities

iii. Axis movements

The most important step in our initial configuration is to ensure that the axis’ of the printer move, and move in the correct direction. 
The axis of the printer is set up in the following: 

[image: image34.png]



The extruder travels in the X axis, as well as up and down in the Z axis, while the print bed travels only in the Y axis. 

The starting point for the printer, the co-ordinates of 0,0,0 is towards the front right corner of the printer. This means that if we move any one axis in a positive direction, we are moving away from this point, and if we are moving in a negative direction, we are moving towards this location. 

This is illustrated in the diagram below. For instance, the maximum travel of the X axis, 140mm would have the extruder all the way to the left side of the print bed, while extending the Y axis to its maximum travel of 200mm would bring it forward. 

  [image: image35.png]



To test our axis’, ensure that each of the axis’ are near the middle of their travel and connect your printer to your PC and follow the previous steps to load a temporary model and open the Pronterface UI in Cura. 

We will test one axis at a time for the following procedure. 

To test the X axis, using the Pronterface controller, click on the second inner ring of the right side button (labeled +X). The X axis should move away from our starting point of 0,0,0. Repeat this step in the opposite direction for the X axis. The axis should move back towards our starting point. 

[image: image36.png]



Repeat this step for the Y and Z axis as well. Both axis’ should move in the correct direction. If for whatever reason they move opposite of the intended direction, disconnect your printer, and unplug the power to the printer. Remove the electronics enclosure cover, and locate the motor driver for the affected axis. Simply unplug the motor cable and reverse itse direction. This will reverse the direction of the motor. Reconnect your power supply and conenct the printer to your PC. Repeat the steps above and validate that the axis’ move in their proper direction. 
iv. End stop validation
With the correct movement of the axis’ verified, we must now verify that our end-stops function as they should. 

To do so, ensure that none of the axis’ are touching the end-stops. From the Pronterface interface, type M119 into the Gcode window and press entered. If all goes well, you should see the Gcode window dispay several lines of text. These should read as:
X_min: open

Y_min: open

Z_min: open

Now, using your free hand, depress the X axis end-stop and while depressed, repeat the previous step. Your response should be:

X_min: TRIGGERED

Y_min: open

Z_min: open

Repeat this procedure for the Y and Z axis. All 3 axis should behave similarly. If any of the end stops behaves differently, disconnect your printer, and verify that the end stop connections on both the end stop and on the RAMPS board are correct. Repeat the procedure to ensure the end stops are working correctly. 



Final Printer Configuration

The steps below will help you fine tune the performance of your printer and prepare it for the first print. 

a. Setting your end stops 

Setting your end-stop distance is a simple yet important step. To complete this step you will need your M3 hex wrench. 
To set the X axis end-stop, slide the carriage to the minimum extent. The X end stop should be depressed by the screw on the extruder assembly. If it is not, use your hex wrench to extend the screw onto the tab of the end stop. You should hear an audible click when the end-stop is depressed. Do not extend the screw any further than this. 

[image: image37.png]



The procedure is much the same for the Y axis as well. Extend the Y axis to the minimum of its travel and check to see if the screw on the bottom of the Y axis is contacting the end-stop. If it is not, use your hex wrench to extend the screw until it does. Again, listen for the audible click of the screw depressing the end-stop. 

[image: image38.png]N

//
AN //
\ \
%A\ 4
,, '/&.’Af« :.x\lul
b/&\; il
Yavy
/,





Adjusting your Z end-stop will be covered in the next chapter. 

b. Leveling your print bed

Leveling your print bed is perhaps the most crucial step we must take before we print our first object. Thankfully it’s a straight forward procedure and one that can be completed easily. You should also periodically re-level your print bed. 
You will need the following:

· Blue painters tape

· A business card or semi-thick sheet of paper

· M3 hex wrench

Before we begin, we must first prepare our print surface. Printing with PLA can be one of the easiest filaments to work with, as well as delivering some of the best results. PLA is also unique in that it does not need a heated bed to print on, and can be printed on a very easy to apply bed, 3M Blue Painters Tape.

Use Blue Painters Tape to cover the print bed completely. Blue Painters tape can usually be found at most hardware stores, in widths ranging from 1 to 2 inches. 
[image: image39.png]mmlroars

£





It is important that when applying the strips of painters tape, to not overlap the tape. Overlapping the tape will produce a small ridge in your print surface which can affect your print quality and results. It’s best to butt each strip up against each other, or leave a very small gap between each strip like so:

[image: image40.png]



With a layer of painters tape applied to the bed, we can now begin to level our print bed. 

First we must set the initial height of the Z axis. To do so, we will connect the printer to our PC and launch Cura and the Pronterface interface as illustrated earlier. 

Move the X axis to its minimum position, along with the Y axis. Using the pronterface interface, you can click on the small Home icons in each of the X and Y controllers to home each axis to its minimum positon. 

With the X and Y positon at their minimum, begin to lower the Z axis. As you get closer to the print bed, lower your Z axis 1mm at a time.

As the extruder tip reaches the print bed, grab your business card, or semi-thick piece of paper. Slide the paper between the print bed and the extruder. Like so:

[image: image41.png]



Continue to lower the Z axis, this time in 0.1mm increments. 

[image: image42.png]



Lower the extruder until you can feel a slight tug when moving the paper between the bed and extruder. We don’t want the print head to be crushed into the paper, just slightly snagging it. The thickness of the thick paper represents an ideal first layer height. 

With the initial layer height set, using your M3 hex wrench, extend the Z Axis screw until it contacts the Z end stop. Listen carefully for the audible click of the end-stop, and stop extending the screw when you hear it. You’ve now set your initial layer height. 

[image: image43.png]



We’ve now set one corner of our print bed as the reference point for the rest of the bed. 

In order to level the remainder of bed, move the extruder to the extent of the X axis (the far left corner) 

[image: image44.png]



Using the method before, slide the card between the bed and the extruder. Using your hex wrench, either tighten or loosen the screw in that corner to raise or lower the print bed until the paper drags between the bed and extruder. 
[image: image45.png]



Continue this procedure for the left rear and right rear corner of the print bed as well. Using your hex wrench and card to set the height of the print bed. 
[image: image46.png]


[image: image47.png]



If all goes well, all four corners of your bed should have the same set distance between the print surface and tip of the extruder. Keep in mind that the print surface should not be level with the surface the printer is sitting on, but be level with the extruder tip at any point of its travel. 
Congrats, you’re one step closer to your first print!

c. Loading Filament

Note: Your printer is compatible with 1.75mm diameter materials, and works best with PLA filament. 

Before we print our first object, we must load filament into the extruder. To do so, mount a roll of filament onto the filament spool holder and find the end of the filament on the spool. Using a pair of scissors, remove the last few millimeters of filament to ensure that there is a clean cut edge entering the extruder. 
Using your fingers, gently straighten out several inches of filament as best as you can. The filament will have a tendency to curl, and straightening the filament will make it easier to insert into the extruder. 

Connect your printer to your PC, and using Cura pre-heat the extruder to 190 degrees. Do not proceed with the next step until your extruder has reached this temperature. 

With the filament prepared, pull back on the idler on the extruder like so:

[image: image48.png]=N\
.\/u

! ._.sul
&Y . =
Jie i
. ~... &7

‘!





While pulling back on the idler, insert the filament between the drive gear and idler bearing and into the hot-end like so: 

[image: image49.png]I .. N
g ] .Wﬂ.\-mi,. P

S




You should be able to feel the resistance when the filament enters the hot end. Let go of the idler and check to ensure that the idler bearing is making contact with the filament. 
d. Extruding Filament

Now that we have pre-heated our extruder and loaded the filament, we will now prime the extruder and check to see that it is extruding filament correctly. 
Open the Pronterface UI from Cura if it is not already open, and check to ensure that the extruder is at its operating temperature of 190 degrees. 

From the control panel, locate the extruder panel. This is marked with two arrows labeled ‘E’ for extruder. Click on the ‘+10’ button to begin extruding. This will extrude 10mm of filament. 

[image: image50.png]L =LDN




You should now see the extruder drawing in the filament. If the extruder is retracting the filament, stop the procedure and disconnect your printer. Open the electronics enclosure and reverse the connection of the extruder motor. Repeat the process above to verify that the motor is operating in the correct direction.  

Keep extruding 10mm of filament. It will take several attempts before you see any material extruding from the extruder. 

It is also important to ensure that the idler is applying adequate pressure to the filament and drive gear. If while extruding you notice the filament slipping or the drive gear not grabbing the filament, tighten the 2 screws on the idler assembly. This will increase the pressure against the extruder drive gear.  

You should now begin to see filament extruding from the extruder. Ensure that the filament is extruding in a thin strand and is not building up around the tip of the nozzle. With your extruder primed and ready, we’re almost ready to begin your first print. 

Note on calibrating your extruder.

In some cases it may be necessary to calibrate your extruder and the amount of filament it extrudes per millimeter. For an excellent introduction to this procedure and a video guide, please visit Thomas Sanladerer’s YouTube video here: https://www.youtube.com/watch?v=YUPfBJz3I6Y
Your default firmware values should provide a decent calibration and good printing results. 

e. Understanding the Printer LCD & SD Card Reader
[image: image51.png];1.126f-=' :

A 7R Y a8 Z1RZ. 6868
elBE:  Sh———x B——I——
MarlePrint U2 readd.




Your printer is equipped with a LCD display, as well as a built in SD Card Reader. This allows your printer to print untethered without the need for a PC. Printing from an SD card will be covered later in this document. The LCD displays a variety of information during a print. The primary fields are the following: 

[image: image52.png]B o L




The first line displays the real time temperature reading of the hot-end, and the desired set temperature.
[image: image53.png]



The second line contains positional data of each of the axis’. 
[image: image54.png]RlBEY: Sh———3% f——i——




The last line of the display contains the flow rate. The flow rate is the amount of plastic the printer is extruding. Typically this is set in firmware or slicing profiles. The final two options report back the total percentage of the model that the printer has printed, as well as the elapsed time. 
The LCD panel is also equipped with a rotary dial. Pressing this dial will activate the Menu tree and give host to a wide variety of options to control and modify your printer. This menu system is broken into several sections with multiple sub sections:

These menus and the additional SD card slot allow your printer to print without an computer attached. 

Top Level:

Watch: This will return you to the main overview screen.

Prepare: This menu contains several sub options for preparing your printer prior to printing

Control: This menu as well as its sub-options allows the user to control aspects of their printer such as temperature, motion and firmware changes (if enabled). 
Card Menu: The last option allows you to browse and load any saved .gcode files if an SD card is inserted. 
Each of these menus contains a sub-menu with several options as well. Below is a brief review of the sub menus.

Prepare:

Disable Steppers: This will disable the motors and allow the user to move each axis by hand. 

AutoHome: This will home all axis to their starting points of 0,0,0.

Preheat PLA: This option will pre-heat the extruder to the specified temperature for PLA

Preheat ABS: This option will pre-heat the extruder to the specified temperature for PLA

Cooldown: Cooldown will disable the heater for the extruder and enable the cooling fan. Handy for after completing a print. 

Move Axis: This menu option and sub options will allow the user to move each axis’ as well as the extruder, a set distance. 

Control:

Temperature: The submenus under Temperature allow the user to define the default settings for extruder temperature, fan speeds, and other settings for both PLA and ABS. 

Motion: The motion menu contains the variables used in defining how the printer behaves during movements. This includes options such as acceleration values, jerk settings and retract/extrude values.

Store Memory: This option allows the user to save any changes they have made in any of the previous or sub menus. 

Load Memory: This option allows the user to load the configuration that is stored in the EEPROM of the printer. 

Restore Failsafe: The last option will reset the printer’s configuration to the default values that were flashed to the firmware. 
Card Menu: Lastly, if one wishes to print directly from the SD card, a user would first save the .gcode file created in their slicing application. After copying the gcode from their PC to the SD card, the user can then load the SD card into the printer. Selecting this menu will display all gcode files present on the card, and selecting any of the gcode files will initiate the print. 



Your First Print

a. Understanding the Cura Interface

Now that we’ve finally configured our printer, we may begin the process of our first print. 
[image: image55.png]! i‘:“-sﬂc| Advanced

Plugins

Start/End-GCode |

Quality

Layer height (mm)
Shell thickness (mm)
Enable retraction

Fill

0.3

0.9
4

Bottom/Top thickness (mm) 0.9

Fill Density (%)

Speed and Temperature
Print speed (mm/s)
Printing temperature (C)
Bed temperature (C)
Support

Support type

Platform adhesion type
Filament

Diameter (mm)
Flow (%)

15

70
205
70

None

None

1.75
100.0




The Cura interface, like discussed earlier is broken into 2 areas, the print settings, and the print stage.

 The print settings contain variables that can be user adjusted to modify the print quality, speed and other options. Please refer to the settings referenced earlier in the guide. The settings panel also contains additional tabs for more control over the print settings. 

[image: image56.png]| Basic AelEqlesel Y Plugins

Start/End-GCode

Machine

Nozzle size (mm)

Retraction

Speed (mm/s) 30
Distance (mm) 2
Quality

Initial layer thickness (mm) 0.3
Initial layer line width (%) 100
Cut off object bottom (mm) 0.0

Dual extrusion overlap (mm) 0.15

Speed

Travel speed (mm/s) 100
Bottom layer speed (mm/s) 35
Infill speed (mm/s) 75
Outer shell speed (mm/s) 65

Inner shell speed (mm/s) 75

Cool

Minimal layer time (sec) 5
Enable cooling fan 4




Under the advanced tab you may see additional options for controlling items such as the (1) extruder nozzle diameter,(2) filament retraction and retraction speed. Under the Quality heading, the user can adjust settings such as the initial layer thickness, and width. The Speed heading contains all the settings relating to the speed of the machine, including its movements between prints, outer speed and infill speed. Lastly the cooling options allow the user to define how the print is actively cooled with the Cooling fan. 

The print stage will display the build volume of your printer, as well as any models or objects that you place on the stage. Users an pan, rotate and zoom around the stage using their mouse. The stage also allows the user to see several views of their model, ranging from an X-ray view to individual layers of the printed model:

[image: image57.png]Netmal| 4

®venhang
AR ay
[Favens





b. Loading and slicing your first model

We’re now ready to slice our first model. With Cura open, click the folder icon in the upper left corner of the print stage.

[image: image58.png]B O




You should now be greeted by a file explorer window. From here select the model you wish to print. For our first print, it is advised that you begin with something simple and quick to print, such as the mapleMaker key fob. (Link here)

[image: image59.png]le Tools Machine Expert Help

Basic [ advanced | Plugin | Start/End-GCode

Quality

Layer height (mm)

Shel thickness (vm)

Enable retraction

Fll
Bottom/Top thickness (m)

Fil Density (%)

Speed and Temperature

Printspeed (nm)s) )

Printng temperature (€) 210

Support

Support type

Platform adhesion type.

Flament

Diameter (nm)

Flow (%)

@ v 1« Maplebrint Mini V2 » STL Vo] | serchst

Organize = Newfolder
Ui MsplePrint Mini 2
Ui Printer_Build_Docs
Ui Keyboard
Ui ps0
5 nventor
Ui Visusl Stucio 2013
b cao

@ OneDrive
& Homegroup

M This PC

File name:

Neme
(<] MPMV2_Crossmember 3.ti

2] MPMV2_Crossmember 4.ti

<] MPMV2_Crossmember 5.ti

2] MPMV2_Crossmember 6.ti

(] MPMV2_Crossmember 7.sti

2] MPMV2_Crossmember 8.ti

(] MPMV2_Extruder Base.stl

(] MPMV2_Extruder_Fan_Mount.st

(] MPMV2_Extruder Mount Platest

(] MPMV2_Extruder Mount Plate E30.st1

] MPMV2_Extruder V2R Base.stl
v«

/13/20158: ¥
>

MPMV2_Extruder Base.stl Al €5t 0" da” amf;" by v

Open

Cancel





Your model will be automatically sliced and processed once it is loaded. You are now ready to print! 

Any changes made to the slicing variables in the right hand window will automatically re-process the model and re-slice the output. 
Should you wish to save the gcode and print from the SD card, disconnect your printer from your PC and click the Save icon. Save the .gcode file to your SD card, and then insert it into your printer.  

c. Starting & completing a print

Once you’ve chosen and sliced your object, ensure that your printer is connected to your PC and that it is connected in Cura. Once ready, simply click the ‘Print with USB’ button in the upper left of the print stage to begin the printing process. Your printer will heat the extruder, and then home each axis. 
Once the extruder reaches its temperature, and all the axis’s are homed, the printer will begin to print its first layer. 

Check to ensure that the filament is feeding correctly, and that the first layer is adhering to the bed. Your first layer should be slightly “squished” and adhere to the blue painters tape. Notice in the image below how the first outside layers are slightly fatter than the rest?
[image: image60.png]



Congratulations! You’re now printing! Soon your first test piece will be finished, and you’ll be able to pull it off the bed! When it is done, use a small putty knife, or scraper to peel it off the bed. Be careful not to tear the tape, although you can reapply more, and should do so periodically. 



Quick Tips
Maintenance Notes: 
Like any machine, your printer will require some periodic maintenance to ensure that it operates correctly. 

· Clean your printer using a light cloth and ensure that the extruder, filament, and print bed are clear of dust or debris

· Check the frame components for loose hardware or worn out parts. If any parts need to be replaced, print a new part and replace the affected pieces.

· Keep the linear rods clean of all debris and periodically add a small amount of machine oil to them to keep them lubricated. Do not oil your hot end or extruder gear. 

· Check all connections and wiring for loose connections or broken wires. 

· When not using your printer for extended periods of time, store your filament in a cool dry location with as little moisture as possible. If possible store your filament with a small silica gel packet to absorb any present moisture. 
Hardware:

Sloppy corners and bad details: If you find your prints corners look more like circles, or that small details in your prints are missing or look less than ideal, check the tension of the belts on your X and Y axis. Sometimes, loose or sagging belts can cause loss of detail and accuracy.
Slipping Filament: Is your filament not extruding correctly, or consistently? Try increasing the compression on the extruder idler by tightening the 2 long screws. This will force the filament into the drive gear and hopefully resolve the slipping. However, if your extruder is working fine, but no plastic is coming out of the hot end, your hot end may be jammed and may require dis-assembly to clean and fix. 

Printer not communicating: If you find your printer is not cooperating with your PC, check to ensure that the serial driver is installed correctly. To do so, uninstall Cura and then re-run the Cura installer. The installer includes the required drivers and should fix any issues you are having. 

Prints not sticking to the bed: If you find that your prints are not sticking to the bed, check your first layer height. The first layer should go down nice and flat, and be a bit fatter than the other layers. If you see any wavy lines or a very very thin extrusion, then chances are your first layer is too high. After several prints you may also need to remove and re-apply fresh blue painters tape to your bed as well. 

Resources:

For more information, tips, and tricks on using your 3D printer, visit some of these awesome sources of information.

3D Printing Resources:

RepRap Wiki: http://reprap.org/
Thomas Sanladerer’s 3D Printing Guides: https://www.youtube.com/user/ThomasSanladerer/
E3D Online: http://e3d-online.com/
3D Model Repositories:

YouMagine: www.youmagine.com
GrabCAD: www.grabcad.com
Makerbot Thingiverse: www.thingiverse.com
3D Modeling Software:

Sketchup: http://www.sketchup.com/
Tinkercad: https://tinkercad.com/
Autodesk 123DCatch: http://www.123dapp.com/catch
Blender: http://www.blender.org/
Meshlab: http://meshlab.sourceforge.net/
Netfabb: http://www.netfabb.com/basic.php
